- SCOPE OF WORK

To establish a pool of vendors for job order road resurfacing and related services projects. Pool vendors shall be notified of projects via the County’s RFQ system as described in Section 4.  Contractors shall be required to possess and maintain a valid Class A or B General Contractor’s License and shall demonstrate comprehensive knowledge of the Florida Department of Transportation (FDOT) Standard Specifications for Road and Bridge Construction, the Florida Department of Traffic Standard Specifications for Road and Bridge Construction, Manual on Uniform Traffic Control Devices (MUTCD), 2025 edition, or the most current edition as amended.

This is an indefinite quantity contract with no guarantee that services will be required. The County does not guarantee a minimum or maximum dollar amount to be expended on any contract(s) resulting from this solicitation. The scope of work represents the minimum standards required. All work performed shall be in strict compliance with the latest codes, standards, and practices and in accordance with Federal, State, and Local laws.

- CONTRACTOR RESPONSIBILITIES

Contractor shall:

  - Provide all labor, fuel, materials, equipment, storage, supplies, and incidental costs necessary to complete the tasks.

  - Provide for skilled, qualified, and English-speaking staff with a cell phone in good working order.

  - Provide adequate personnel, in the event of sickness or any absence, a substitute of equal skill shall be provided at no additional cost.

  - Provide a neat and clean in appearance dress code (as applicable) for Contractor’s employees that consists of a shirt with company name, pants, and work shoes/boots and all associated PPE such as appropriate class safety vest, safety glasses, etc.

  - Project a professional image, deal effectively with the public, and discharge duties in a courteous and efficient manner.

  - Provide proof of chemical applicators’ licensure prior to any Chemical treatment utilizing herbicides for vegetation removal.

  - Maintain the project work site in an attractive, and uniformly clean manner which will reflect favorably upon the County and the Contractor.

  - Perform work at a time that minimizes disturbance or interference to the flow of traffic.

  - Correct all apparent deficiencies within three (3) calendar days of any work that fails to conform to the specifications.

- COUNTY RESPONSIBILITIES

County will:

  - Reserve the right to award to one or more vendors.

  - Reserve the right to add or remove services in conjunction with the County’s needs.

  - Reserve the right to inspect and approve all material, supplies, workmanship, and equipment for contract performance.

  - Reserve the right to dismiss Contractor’s staff for disorderly conduct or unsatisfactory performance in accordance with contract specifications.

- DELIVERY REQUIREMENTS AND ACCEPTANCE.

  - When it is determined that a road resurfacing project is needed, the County will issue a Request for Quote (RFQ), utilizing the County’s RFQ System.

  - The RFQ will define the scope of work, project limits, pre-bid meetings (if any), calendar days to complete the project, and submittal requirements.

  - The project limits will be marked with pink or white paint.

  - Contractor shall perform a site visit and inspect all sites prior to the question submission deadline specified within the RFQ.

    - Contractor shall document items, including but not limited to, additional milling, leveling, course and radius paving, discrepancies in quantities, or project limits, etc.

    - Contractor shall submit questions to the County via e-mail prior to the question submission deadline. Questions received after this time will not be answered.

    - An addendum will be issued for all questions received and posted to the RFQ.

4.5.	Contractor shall submit a bid response prior to the bid closing date.

4.6.	The submitted bid response constitutes the Contractor’s agreement to all quantities and associated costs specified in the RFQ.

4.7.	Change orders shall be issued for items that have been added to or deducted from the original RFQ. Change orders may be subject to bituminous and fuel adjustments in accordance with the terms of this contract.

4.8.	A price redetermination will not be considered for projects taking less than 90 production days, or any projects exceeding original project days plus any approved added weather days.

- TECHNICAL REQUIREMENTS

All work and materials shall conform to the Florida Department of Transportation (FDOT) Standard Specifications for Road and Bridge Construction, Florida Department of Traffic Standard Specifications for Road and Bridge Construction, Manual on Uniform Traffic Control Devices (MUTCD), 2025 or the latest edition. Relevant FDOT reference sections are indicated under the corresponding items listed below.

# 5.1. Public Notification of Work

5.1.1.	Reference FDOT Standard Specifications Section 990, TEMPORARY TRAFFIC CONTROL DEVICE MATERIALS

5.1.2.	Contractor shall notify all residents within the project work area and project schedule and explain the expected level of inconvenience associated with the project.

5.1.3.	Contractor shall provide public notification to all residences and parked vehicles on the roadway within the project limits seven (7) calendar days prior to the commencement of work.

5.1.4.	Contractor shall obtain County approval prior to placement on residences and vehicles parked on neighborhood roads within the project limits.

5.1.5.	Contractor shall install PCMS on all major roadways and industrial parks to display lane closure information including, but not limited to, anticipated lane closure, closure extent (i.e. “Next 3 Miles”), and daily hours of closures.

5.1.6.		All costs associated with the PCMS shall be charged per unit/per day and include the installation, removal, mobilization, MOT, equipment, labor, and any other incidental charges associated with the operation.

5.1.7.	The Contractor shall make every effort to avoid blocking access to businesses for an extended period and provide the business with written notice, approved by the County, at least forty-eight (48) hours prior to access restriction.

5.1.8.	Contractor shall minimize the impact to school related traffic during before and after school hours when project limits are within the vicinity of a school,

5.1.8.1.	It shall be Contractor’s responsibility to coordinate a pre-construction meeting with the County, an official from the affected school, and Contractor’s Project Superintendent.

5.1.8.2.	An adjusted work schedule will be established, as a result of the pre- construction meeting with the school official.

5.1.8.3.	Portable Changeable Message Signs (PCMS) will be required for seven (7) calendar days prior to the commencement of the work to provide public notification of the upcoming work.

## 5.2.	Shoulder Preparation

## 5.2.1. 	Vegetation shall be cut back six inches (6”) from the edge of pavement, including any present on the pavement surface, and removed prior to the placement of new asphaltic concrete. Any areas where excessive material has been removed shall be promptly repaired by the Contractor.

## 5.2.2.	Shoulder drop-off shall not exceed one (1) inch in depth, as measured from the top of the asphaltic concrete at the pavement edge.

## 5.2.3.	If an excessive drop off is created, it shall be restored to no more than one (1) inch as described above.

## 5.2.4.	If an area is to be left overnight with the excessive drop off, barricades marking the hazard shall be installed in accordance with FDOT specifications.

## 5.2.5.	The cost for shoulder preparation (including mobilization, Maintenance of Traffic (MOT), equipment, labor, and all associated activities) shall be incorporated into the price for installing new asphaltic concrete.

## 5.3.	Vegetation Removal

5.3.1.	Vegetation in the cracks or joints shall be removed by using a propane torch or a chemical herbicide and subject to approval of the County. The vegetation shall be totally browned before the new asphaltic concrete is installed.

5.3.2.	Chemical treatment shall not be applied without providing proof of chemical applicators licensure.

5.3.3.	Chemical treatments shall be applied according to the manufacturer’s specifications to ensure public safety and a log of chemical treatments shall be kept with a copy supplied to the County’. The log shall contain information of the chemical product, manufacturer, application rate, application location, application date and time, safety data sheets (SDS), and weather conditions at the time of application.

5.3.4.	The cost of removing vegetation by either propane torch or chemical herbicide treatment (including mobilization, MOT, herbicide, labor, etc.), and shall be incorporated into the price of the paving operation.

## 5.4.	Road Base Repairs / Heavy Milling Repairs

5.4.1.	Reference FDOT Standard Specifications Section 334 SUPERPAVE ASPHALTIC CONCRETE.

5.4.2.	Full depth milling repairs shall be provided to areas of roadway with failed base.

5.4.2.	County shall mark all areas for base repairs with white and pink paint.

5.4.3.	Marked areas shall be milled to provide square or rectangular shaped repairs to a depth of six (6) inches and replaced with six (6) inches of Type SP 12.5 asphaltic concrete in three (3), two (2) inch lifts.

5.4.4.	Jagged or misshaped repairs shall be rejected and repaired at no additional cost.

5.4.5.	The unit cost for base repairs and/or heavy milling shall include, but not limited to mobilization, MOT, milling, asphalt, etc.

5.5.  Milling of Existing Asphaltic Concrete Pavement

5.5.1.	Reference FDOT Standard Specification Section 327 MILLING OF EXISTING ASPHALT PAVEMENT.

5.5.2.	The cold milling process to remove existing asphaltic concrete may be requested to improve ride quality, correct cross slope, lower the finished grade adjacent to existing curbs, or remove the existing pavement.

5.5.3.	Areas shall be marked in pink or white paint, milled to the depth specified, and milled to the outside edge of the painted area.

5.5.4.	Contractor shall notify the County of any traffic light advance warning loops that will be affected forty-eight (48) hours before the milling operation starts.

5.5.4.1.	Contractor will not be held responsible for damage of loops affected by the milling operation.

5.5.5.	The milling machine shall be equipped to effectively limit the amount of dust escaping during the milling operation. Pre-wetting of the pavement may be required if it is determined there is too much dust being created.

5.5.6.	Profile correction milling may be required on specific portions of roadways and will be paid based on an average milling depth of zero (0) to two (2) inches. Any area which exceeds the two (2) inch average depth will be compensated at the appropriate unit cost for total depth.

5.5.7.	Asphalt millings generated shall be hauled from the project location at no additional cost to the County, except for millings that the County directs the Contractor to haul to three designated area(s) not exceeding 25 miles from the project.

5.4.6.1. 	Site 1 – Maintenance Area 1, 2310 W. Griffin Rd., Leesburg, FL 34748

5.4.6.2. 	Site 2 – Maintenance Area 2, End of Allegiance Ct., Groveland, FL 34736

5.4.6.3. 	Site 3 – Maintenance Area 3, 19720 E. Fifth St., Umatilla, FL 32784

5.5.8.	If the County elects the Contractor to haul asphalt millings to one of the three designated County sites, payment will be made only for the actual cubic yards hauled.

5.5.8.1.	Truck loads should average 18 cubic yards each.

5.5.8.2.	Load tickets will be issued at the project site and collected by the County at the designated dump site.

5.5.8.3.	The unit cost for milling shall cover mobilization, Maintenance of Traffic (MOT), milling equipment, labor, and all related expenses.

5.5.9.	County reserves the right to insert County dump trucks into the milling operation at any time and to retain ownership of the resulting millings. If the County exercises this option, it shall be done in a manner that does not impede the Contractor’s production and at no additional cost to the County.

5.6.  	Prime and Tack Coats

5.6.1.	Reference FDOT Standard Specification Section 300, PRIME AND TACK COATS.

5.6.2.	A tack coat shall be installed prior to the installation of the new asphaltic concrete.

5.6.3.	The tack coat shall only be applied to one lane of traffic at a time, and shall not exceed one (1) mile, unless otherwise determined by the County.

5.6.4.	The tack coat shall be applied only to areas scheduled to receive an asphalt layer the same day of operation and shall be protected from all traffic until the asphalt has been placed.

5.6.5.	The unit cost for tack coat shall be included in the installation of new asphaltic concrete. No mobilization shall be charged for this operation and the cost provided shall include, but not be limited to: MOT, equipment, labor, etc.

5.7.	Asphaltic Concrete

5.7.1.	Reference FDOT Standard Specification Section 320, HOT MIX ASPHALT PLANT METHODS AND EQUIPMENT and Section 330, HOT MIX ASPHALT GENERAL CONSTRUCTION REQUIREMENTS.

5.7.2.	Contractor shall be responsible to ensure positive drainage of the surface from the roadway to curb, edge of roadway and/or valley gutter.

5.7.3.	There shall be no standing water along the pavement where there is no standing water in the curb.

5.7.4.	Contractor shall provide proper and adequate fall across entrances and cul-de-sacs to ensure proper drainage of these areas.

5.7.5.	The crown of the roadway shall be re- established to ensure a 2% fall from the center of the roadway to the outside edge of the roadway or be placed to match existing cross slope. No water shall be standing in the crown of the roadway.

5.7.6.	Should there be low areas that may result in water standing on the new pavement, a resolution shall be discussed with the County and resolved prior to starting work in the area.

5.7.7.	The unit cost shall be based on one (1) inch increments. The cost for fractional amounts shall be invoiced according to the actual fractional amount installed.

5.7.8.	The unit cost for asphaltic concrete types FC-5, FC-9.5, FC-12.5, FC-12.5 w/76-22, SP-9.5, and SP-12.5 shall be included in the price for installing new asphaltic concrete. No mobilization shall be charged for this operation, and the cost provided shall include, but not be limited to: MOT, equipment, labor, etc.

## 5.8.	Asphaltic Concrete – Friction Course

## 5.8.1.	Reference FDOT Standard Specifications Section 337, ASPHALTIC CONCRETE FRICTION COURSES.

## 5.8.2.	The County may require the use of:

## 5.8.2.1.	Type FC-9.5 asphaltic concrete friction course.

## 5.8.2.2.	Type FC-12.5 asphaltic concrete friction course.

## 5.8.2.3.	Type FC-12.5 with a 76-22 modified polymer asphaltic concrete friction course.

## 5.8.3.	The unit cost for asphaltic concrete friction courses shall be included in the proposal price for installing new asphaltic concrete. No mobilization shall be charged for this operation, and the cost provided shall include, but not be limited to: MOT (unless allowed), equipment, labor, etc.

## 5.9. Asphaltic Concrete – Superpave

## 5.9.1.	Reference FDOT Standard Specification Section 334 SUPERPAVE ASPHALT CONCRETE.

## 5.9.2.	The County may require the use of:

## 5.9.2.1.	Type SP-9.5 asphaltic concrete

## 5.9.2.1.	Type SP-12.5 asphaltic concrete.

## 5.9.3.	The unit cost for asphaltic concrete Type SP asphaltic concrete shall be included in the price for installing new asphaltic concrete. No mobilization shall be charged for this operation, and the cost provided shall include, but not be limited to: MOT (unless allowed), equipment, labor, etc.

## 5.10.	Leveling Course

5.10.1.	Reference FDOT Standard Specification Section 330, HOT MIX ASPHALT GENERAL CONSTRUCTION REQUIREMENTS.

5.10.2.	Contractor may be asked to install a leveling course of asphaltic concrete over an existing asphaltic concrete road surface. This shall only be utilized at the discretion of the County unless the need for such leveling was established prior to issuance of the Notice to Proceed by Contractor.

5.10.3.	When a leveling course is requested, the specified asphaltic concrete mix is to be placed on the existing irregular pavement or base, the leveling course shall be used to help provide a smooth driving surface by filling in voids and deformations such as wheel ruts, depressions, etc.

5.10.4.	When a leveling course is requested, the estimate shown on the Project Order Form shall be based on one-half (1/2) inch for the specified area of the road surface. It is understood that some areas of the roadway will not require the entire one-half (1/2) inch and other areas may require greater than one-half (1/2) inch of asphaltic concrete. There should be no additional charge to the County for additional asphaltic concrete needed nor shall the County expect a credit if the specified amount of asphaltic concrete was not used. Contractor shall ensure that the placement is completed so that the final product provides a smooth driving surface.

5.10.5.	Profiling of the roadway to change the grade or slope of the road shall be done with the use of asphalt and/or a milling machine.

5.10.6.	If the County chooses to change the profile of the road using asphaltic concrete, it shall be quantified and paid by using the unit prices of designated asphaltic concrete unit cost in Attachment 2 – Pricing Sheet.

## 5.11	Radius Paving

5.11.1.	Contractor shall overlay designated roadway intersections that connects to any project road.

5.11.2.	Paving will be to a distance of fifteen (15) feet, or as determined by the County, from the intersection (see diagram below).

5.11.3.	The limits of the radius area shall be marked with white or pink paint. Not all intersections shall be paved. If there are not marks, it shall be assumed that the EOP be followed through the intersection or treated as a driveway for overlay applications.

5.11.4.	Existing pavement markings within the radius area being paved shall be replaced to the specifications outlined.

5.11.5.	The joint between the existing and new pavement shall provide a smooth transition.

15’

5.11.6.	The square yardage for the radius paving shall be calculated and added to the total square yards as stated on the Project Order Form.

## 5.12.	Driveways

5.12.1.	Driveway joints with the exception of concrete driveways and brick pavers, are to be paved with a one (1) foot per inch or greater slope to achieve a smooth transition.

5.12.2.	Driveways shall be swept and tacked before paving.

5.12.3.	The cost of installing asphaltic concrete on driveway aprons shall be considered incidental and shall be included in Contractor’s per unit resurfacing cost and no additional cost shall be permitted for driveway paving.

## 5.13.	Compaction

5.13.1.	Reference FDOT Standard Specification Section 330, HOT MIX ASPHALT – GENERAL CONSTRUCTION REQUIREMENTS.

5.13.2.	If the County feels an area does not meet these requirements, they shall hire an independent testing laboratory to determine accordance with this specification.

5.13.3.	Areas not in conformance with this specification will need to be removed and replaced at Contractor’s expense.

## 5.14.	Smooth Transition

5.14.1.	The Contractor shall install a five (5) foot tapered keyway mill on all adjacent roads that are not being milled, in order to ensure a smooth transition between the new and existing pavement.

5.15.	Quality Inspections

5.15.1.	Core samples shall be taken by an independent laboratory approved by the County.

5.15.2.	Contractor shall provide the County with the address and contact information of the laboratory prior to any resurfacing work starting.

5.15.3.	Core samples shall be taken every five hundred (500) feet regardless of how many lanes are being resurfaced.

Core

5.15.4.	Core samples shall be taken in a random pattern as indicated on the diagram below.

5.15.5.	Roads that are less than one thousand (1,000) feet shall have a minimum of two (2) core samples taken.

5.15.6.	The cores shall be taken at a depth that will show the new and existing asphaltic concrete.

5.15.7.	Holes that were created by taking the core samples shall be filled with a product approved by the County.

5.15.8	A determination of the average thickness of the asphaltic concrete shall be made from the samples by the testing laboratory and the results shall be sent from the laboratory directly to the County.

5.15.9	If the Contractor wishes to obtain additional core samples beyond those required by the County, such requests must be submitted in writing, and all associated costs shall be borne by the Contractor.

5.16. Deficiencies of Asphaltic Concrete Thickness and Ride Quality

5.16.1.	The County will permit a deficiency in the thickness of the asphaltic concrete overlay not to exceed one-quarter (1/4) inch.

5.16.2.	The Contractor shall utilize an asphalt transfer machine approved by the County for all roadway resurfacing involving milling, unless a written exception is granted by the County. The County shall not permit the commencement of work without the required transfer machine. In the event the transfer machine becomes inoperable during production, the County may, at its sole discretion, allow work to continue for the remainder of that production day.

5.16.3. When the deficiency in the thickness of the asphaltic concrete exceeds one-quarter (1/4) inch but does not exceed three-eighths (3/8) inch of the specified thickness, the County shall permit the Contractor to leave the asphaltic concrete in place without any compensation.

5.16.4.	County shall determine the affected square yard area by multiplying the lane width by the length between the nearest acceptable core samples, and no payment shall be made for this area.

5.16.5.	When the deficiency in thickness exceeds three-eighths (3/8) inch of the specified thickness, the Contractor shall correct the deficiency. In all cases where the installed asphaltic concrete is deficient beyond this limit, the Contractor shall restore the deficient area by applying new asphaltic concrete.

5.16.6.	The minimum repair thickness must comply with the layer thickness standards specified under the asphaltic concrete section of these specifications.

5.16.7.	The Contractor shall replace the full thickness as directed by the County, extending at least fifty (50) feet beyond each end of the deficient area.

5.16.8.	County will mark the area to be corrected with white or pink paint, and the paving shall extend to the outer edge of this paint mark.

5.16.9.	Any additional costs incurred to correct deficiencies shall be the sole responsibility of the Contractor. No additional charges shall be made to the County, even if the repair results in the final thickness of the asphaltic concrete overlay exceeding the originally specified thickness.

5.16.10.	Asphalt smoothness shall be tested at the discretion County on any roadway which included milling.

5.16.11.	Any portion of any roadway associated shall be required to be compliant with Section 330, SURFACE REQUIREMENTS for any lateral or transverse pavement joint provided by Contractor. Any deficiency in excess of three sixteenths (3/16) inches shall be removed and replaced fifty (50) feet in either direction of the deficient area, at no additional cost to the County.

## 5.17.	Manholes / Valves

5.17.1.	Reference FDOT Standard Specifications Section 425, INLETS, MANHOLES, AND JUNCTION BOXES.

5.17.2.	Manholes and water valves shall be adjusted through coordination with the appropriate utility provider to within one-half (1/2) inch of the final surface of the newly installed asphaltic concrete. This shall be accomplished prior to placing the new asphaltic concrete.

5.17.3.	Contractor shall perform utility coordination required to address all manholes/valves located within the project limits. It shall be the utility provider’s responsibility to furnish and install any required adjustments.

5.17.4.	No mobilization shall be charged for this operation, and the cost provided shall include, but not be limited to: MOT, equipment, labor, etc.

## 5.18.	Traffic Stripes and Markings

5.18.1.	Reference Standard Specifications FDOT Section 710, PAINTED PAVEMENT MARKINGS and Manual on Uniform Traffic Control Devices (MUTCD), 2025 or the latest edition.

5.18.2.	If the Contractor is unable to install the pavement markings as prescribed, appropriate traffic control measures shall be provided, such as but not limited to, warning signs, channelizing devices, and delineation to indicate the required travel ways in temporary traffic control zones.

5.18.2.1. 	If Contractor wishes to provide MOT in another manner, the plan shall be submitted in writing and be approved by the County prior to implementation. Lake County shall accept only water borne non-lead type paint.

5.18.3.	Contractor should pay special attention to Section 710-4.3 concerning the retroreflectivity. The minimum retroreflectance of white pavement markings shall not be less than 300 mcd/lx-m2 and the minimum retroreflectance of yellow pavement markings shall not be less than 250 mcd/lx-m2.

5.18.4.	The existing striping layout shall be recorded prior to the commencement of work. The same striping layout shall be installed on the new asphaltic concrete unless the County supplies Contractor with a new striping pattern.

5.18.5.	Striping shall be maintained throughout the duration of the work and shall be in place at the end of each workday.

5.18.6. Painted traffic stripes and markings shall be installed prior to the removal of the MOT.

5.18.7.	If the Contractor fails to install the correct striping pattern, the Contractor shall remove the markings at their own expense using a method approved by the County.

5.18.8.	If the road surface is damaged during the removal process, Contractor shall repair the road surface at Contractor’s expense.

5.18.9.	Hand liners shall be used only for transverse, taper, or gore sections of pavement striping and markings. Hand liners shall not be used for long line pavement stripes that are longer than two hundred (200) linear feet unless the stripes are part of a taper or gore area or an intersection lane line that cannot be installed with a truck mounted applicator.

5.18.10. Contractor shall self-inspect all road markings using the August 7, 2023, Florida Method of Test for Traffic Striping Retroreflectivity Designation: FM 5-543 (or latest edition), to test and certify width, thickness, color, and retroreflectivity.

5.18.11.	Contractor shall submit the results to the County on a reporting form pre-approved by the County.

5.18.12.	County shall review the submitted test results and if the County deems necessary, conduct their own test on any portion of the completed road using August 7, 2023, Florida Method of Test for Traffic Striping Retroreflectivity Designation: FM 5-543 (or latest edition), to test and certify retroreflectivity, width, thickness and color. County shall notify Contractor of any deficiencies.

5.18.13.	Contractor shall correct all deficiencies before final acceptance and payment is made. The County’s tests shall be final and binding.

5.18.14.	All striping operations shall include, but not be limited to: mobilization, MOT, equipment, labor, and any other incidental charges associated with the operation.

## 5.19.	Thermoplastic Traffic Stripes and Markings

5.19.1.	Reference FDOT Standard Specification Section 711, THERMOPLASTIC PAVEMENT MARKINGS. Contractor should pay special attention to Section 711-4.3 concerning the retroreflectivity.

5.19.2.	If the Contractor is unable to install the pavement markings as prescribed, the Contractor shall provide suitable traffic control measures in accordance with the Manual on Uniform Traffic Control Devices (MUTCD), 2025 Edition or the latest edition. These traffic control measures shall include, but are not limited to, warning signs, channelizing devices, and delineation to clearly indicate the required travel ways within temporary traffic control zones.

5.19.2.1.	If Contractor wishes to provide MOT in another manner, the plan shall be submitted in writing and be approved by the County prior to implementation. Lake County shall accept only water borne non-lead type paint.

5.19.3.	Thermoplastic Pavement Markings shall not be installed prior to fourteen (14) calendar days following paving.

5.19.4.	The minimum retroreflectance of white pavement markings shall not be less than 450 mcd/lx-m2 and the minimum retroreflectance of yellow pavement markings shall not be less than 350 mcd/lx-m2.

5.19.5.	In addition to the FDOT Specifications, thermoplastic striping shall not be applied prior to a thirty (30) day cure period from the time of asphaltic concrete application, unless otherwise approved by the County.

5.19.6.	The existing striping layout shall be recorded prior to the resurfacing activity is started. The same striping layout shall be installed on the new asphaltic concrete unless the County supplies Contractor with a new striping pattern.

5.19.7.	If Contractor does not install the correct striping pattern, it shall be Contractor’s responsibility to remove the markings by the method approved by the County.

5.19.8.	Contractor shall be responsible for the cost of the removal and replacement of the correct pattern. If the road surface is damaged during this removal process, Contractor shall be responsible to repair the road surface to the County’s satisfaction at Contractor’s expense.

5.19.9.	Hand liners shall be used only for transverse, taper, or gore sections of pavement striping and markings. Hand liners shall not be used for long line pavement stripes that are longer than two hundred (200) linear feet unless the stripes are part of a taper or gore area or an intersection lane line that cannot be installed with a truck mounted applicator.

5.19.10. Contractor shall self-inspect all road markings using the August 7, 2023, Florida Method of Test for Traffic Striping Retroreflectivity Designation: FM 5-543 (or latest edition), to test and certify width, thickness, color, and retroreflectivity. Contractor shall submit the results to the County on a reporting form pre-approved by the County.

5.19.11.	County shall review the submitted test results and if the County deems necessary, conduct their own test on any portion of the completed road using the same method, to test and certify retroreflectivity, width, thickness and color.

5.19.12.	County shall notify the Contractor of any deficiencies. Contractor shall correct all deficiencies before final acceptance and payment is made. The County’s tests shall be final and binding.

5.19.13.	Longitudinal pavement markings are subject to a 180-day observation period under normal traffic. The observation period shall begin with the satisfactory completion and acceptance of the work.

5.19.13.1. 	The markings shall show no signs of failure due to blistering, excessive cracking, chipping, discoloration, poor adhesion to the pavement, loss of reflectivity or vehicular damage.

5.19.13.2. 	The retroreflectivity shall meet the initial requirements of 711-4.3.

5.19.13.3. 	Contractor shall replace, at no cost to the County, any thermoplastic pavement markings that do not perform satisfactorily under traffic during the 180-day observation period.

5.19.14	All striping operations shall include, but not be limited to: mobilization, MOT, equipment, labor, and any other incidental charges associated with the operation.

## 5.20.	Worksite Traffic Supervisor

5.20.1.	Reference FDOT Standard Specifications Section 102, Maintenance of Traffic.

5.20.2.	Contractor shall have a worksite traffic supervisor that shall be responsible for all MOT by installing and maintaining all traffic control devices.

5.20.3. The worksite traffic supervisor shall review the project on a day-by-day basis as well as being involved in all changes relating to traffic control devices and traffic patterns.

5.20.4.	This person shall handle traffic related situations and have access to all resources needed to maintain traffic control. This person shall be available in case of emergencies twenty-four (24) hours per day and shall be able to respond to the site within forty-five (45) minutes after notification.

5.20.5.	Failure of this person to comply with the provisions of Section 102, shall be grounds for this person being removed from the project. If the County removes this individual from the project, Contractor shall provide a replacement with a person that is properly trained.

5.20.6.	Failure to maintain a designated worksite traffic supervisor shall result in temporary suspension of all activities except MOT, erosion control, and other activities deemed to be necessary for project maintenance and safety.

5.20.7.	The cost associated with the worksite traffic supervisor shall be included in the overall cost of all the operations needed to complete the work as outlined within these specifications.

[The remainder of this page intentionally left blank]