- SCOPE OF WORK

Contractor shall provide all labor, materials, equipment, design, fabrication, installation, and testing required to retrofit two (2) existing DOT-407 stainless steel semi-trailer tankers for use as frontline water tender apparatus.

  - Work shall include two separate retrofit projects:

    - Project 1: 6,000-Gallon DOT-407 Trailer Retrofit (Primary Project).

    - Project 2: 6,000 -Gallon DOT-407 Trailer Conversion from Hydraulic “Wet System” to Standalone Pump System (Secondary Project).

      - Project 2 shall be authorized only after the satisfactory completion and acceptance of Project 1.

- CONTRACTOR RESPONSIBILITIES

  - Contractor shall provide a turnkey solution, including design, fabrication, installation, integration, and testing of all systems that meets or exceeds NFPA standards and FDOT regulations.

  - All work shall comply with applicable standards, including but not limited to:

    - NFPA 1901,1911 (as applicable to water tenders).

    - DOT/FDOT regulations.

  - All components shall be new, unused, and of current production.

  - Contractor shall ensure compatibility with existing trailer configuration and maintain structural integrity.

  - Final product shall be delivered fully operational and tested.

  - Contractor shall provide complete documentation including:

    - Wiring diagrams.

    - Plumbing schematics.

    - Operator manuals.

- PROJECT 1: PRIMARY RETROFIT

  - 6000-Gallon DOT-407 Stainless Steel Semi-Trailer Tank Retrofit

  - Pump & Plumbing System (Fabrication Required)

Contractor shall design, fabricate, and install a complete fire pump and plumbing system including:

    - Fire Pump Installation.

      - Install one (1) PFP-56hp-DSL Fire Pump (or approved equivalent) mounted at the rear of the trailer.

      - Pump performance shall meet or exceed:

        - 400 GPM @ 150 PSI.

        - 575 GPM @ 100 PSI.

        - 700 GPM @ 50 PSI.

      - Provide a full-function control panel with all necessary gauges, controls, and indicators.

      - Provide and install a 30-gallon dedicated fuel tank for standalone pump.

      - Provide and install an isolated 12-volt battery system for pump operation.

    - Plumbing System.

      - Design and install complete intake and discharge plumbing comparable in function and layout to Lake County Fire Rescue (LCFR) Tanker 78.

      - Include all valves, piping, fittings, and supports required for fireground operations.

      - System shall emphasize durability, ease of use, and maintenance accessibility.

    - Gravity Discharges.

      - Install two (2) 12-inch butterfly valves located center-left and center-right.

    - Additional Components.

      - Install one (1) five-way manifold valve with mounting bracket.

      - Provide and install turbo draft system with mounting bracket.

  - Electrical System & Wiring.

    - Provide complete electrical system installation including:

      - All wiring, harnesses, and connections.

      - Proper circuit protection.

      - Weather-resistant, fire service–grade components.

    - Ensure integration between trailer and tractor systems where required.

  - Emergency & Scene Lighting.

    - Trailer Lighting.

      - Install Whelen emergency warning lighting system.

      - Install six (6) Pioneer Plus LED floodlights.

    - Tractor Lighting.

      - Install four (4)-zone NFPA-compliant Whelen system.

      - Install Whelen Duo light bar with WCX.

    - Cab Equipment.

      - Install cab console including:

        - Whelen light and siren controller.

        - Two (2) 100-watt speakers.

        - Power port for mobile UHF radio (County supplied).

  - Water Level Monitoring.

    - Install Tank Vision Pro LED system.

      - Three (3) displays (left, right, rear).

  - Storage & Equipment Mounting.

    - Provide and install tool storage compartments:

      - Weather-resistant.

      - Securely mounted.

      - Durable and accessible.

- PROJECT 2: SECONDARY RETROFIT

  - DOT-407 Trailer Conversion – Hydraulic “Wet System” to Standalone Pump System.

  - Project 2 is an existing DOT-407 trailer currently equipped with a hydraulic-driven (“wet system”) pump.

  - Contractor shall remove or decommission the hydraulic system and convert the unit to a self-contained, engine-driven standalone fire pump system consistent with Project 1.

  - Notice to Proceed (NTP) for Project 2 will not be issued until Project 1 has been completed and accepted by LCFR.

  - Scope of Conversion Work.

    - System Removal / Decommissioning.

      - Remove or safely decommission existing hydraulic pump system, including:

        - Hydraulic pump components.

        - Associated plumbing, hoses, and controls.

      - Ensure all removed systems are properly capped, sealed, and made safe.

    - Standalone Pump System Installation.

      - Install one (1) PFP-56hp-DSL Fire Pump (or approved equivalent), matching the configuration of Project 1:

        - Rear-mounted configuration.

        - Identical or functionally equivalent performance specifications.

      - Provide:

        - Full-function control panel.

        - Dedicated fuel system.

        - Independent battery system.

    - Plumbing Modifications.

      - Modify or replace existing plumbing as required to:

        - Integrate the new standalone pump system.

        - Match operational functionality of Project 1.

      - Ensure compatibility with existing tank structure and discharge configuration.

    - System Standardization

      - The completed unit shall be operationally consistent with Project 1 to the greatest extent possible, including:

        - Pump controls and layout.

        - Discharge configuration.

        - Operational interface for personnel.

  - Electrical & Integration.

    - Provide electrical system modifications as required to support the standalone pump system.

    - Ensure safe removal or isolation of hydraulic system electrical components (if applicable).

  - Testing & Acceptance.

    - Same testing and acceptance criteria as Project 1, including:

      - Pump performance verification.

      - Leak testing.

      - Full operational testing.

- FABRICATION & INSTALLATION REQUIREMENTS

  - All fabrication shall follow industry best practices for structural integrity and corrosion resistance.

  - Components shall be securely mounted for emergency response conditions.

  - Plumbing and electrical routing shall be neat, protected, and clearly labeled.

  - Contractor shall coordinate with LCFR for layout approval prior to fabrication (if required).

- TESTING & ACCEPTANCE

  - Contractor shall conduct full functional testing prior to delivery:

    - Pump performance.

    - Plumbing integrity.

    - Electrical systems.

    - Lighting and controls.

  - LCFR reserves the right to witness testing and perform final inspection.

  - All deficiencies must be corrected prior to acceptance.

- TRAINING & DOCUMENTATION

Contractor shall:

  - Provide on-site operational training for LCFR personnel.

  - Provide complete documentation:

    - Operation manuals.

    - Maintenance manuals.

    - Electrical schematics.

    - Plumbing diagrams.

- WARRANTY

  - Contractor shall provide a minimum one (1) year warranty on all parts and workmanship.

    - All manufacturer warranties shall be passed through to LCFR.

- DELIVERY

  - Completed units shall be delivered to:

Lake County Fire Rescue

20423 Independence Blvd.

Groveland, FL 34736

  - Contractor shall clearly specify delivery timelines for:

    - Project 1.

    - Project 2 (post-acceptance authorization).

[End of Exhibit A]