Contractor shall install asphalt, milling of asphalt, asphalt repairs, sealcoating, limerock base material installation, 8” ribbon curb installation, and install/apply striping for various areas throughout Lake County. Work may occur at various county-owned facilities throughout Lake County.

Contractor will be notified of potential projects and attend a preliminary walk-through with the County’s designated Project Manager. Measurements and quantities will be agreed to at that time.  Contractor will then provide a proposal/quote based on the agreed quantities and prices stated within the contract within three days of the walk-through meeting. Upon acceptance of the proposal/quote, a subsequent purchase order will be issued authorizing the work. In the event of a multiple contract award, the County reserves the right to select the contract that reflects the best value to the County (price and other factors considered).

- CONTRACTOR RESPONSIBILITIES

Contractor shall:

  - Be licensed and fully competent in all aspects of asphalt repairs/installation, limerock base installation, sealcoating, striping, ribbon curb installation, and painting of parking stops in a safe manner.

    - Employ only skilled, qualified workers.

  - Provide all-inclusive quotes to provide 100% turnkey projects that include common installation, repairs, preventative maintenance, and replacement of fixtures/equipment.

    - Include all required labor, material, equipment, plans, engineering, surveys, permitting and local and state inspections.

    - Maintenance of traffic (MOT), when identified as necessary for a project,  shall be the responsibility of the Contractor,  be included as part of the bid price, and shall conform to F.D.O.T.’s most current editions of “Roadway and Traffic Design Standards” for Design, Construction, and Maintained Systems and the Federal Highway Administration (FHWA) “Manual on Uniform Traffic Control Devices (MUTCD) for Streets and Highways.”

    - Include costs for general housekeeping and work area cleaning.

    - Include travel time.

    - Change orders shall not be issued for incidental items or tasks that should have been reasonably construed to be part of the project by the Contractor.

  - Obtain licenses, permits, and fees (including inspection fees) as required to comply with all laws, ordinances, regulations, and code requirements applicable to complete projects.

  - Be responsible for inspections, penalties, fees, or fines for projects.

  - Be responsible for damages caused as the result of completing projects.

  - Furnish all tools and equipment (possibly cranes, lift trucks, boom trucks, cherry pickers, etc.) to complete projects timely.

  - Provide all required labor, supervision, equipment, material, necessary maintenance of traffic (MOT), local and state inspections to provide a 100% completed project.

  - Upon notification by Contractor that the work is complete, the designated County Project Manager shall perform an inspection within five (5) business days of notification of completed work to review the work with the Contractor’s authorized representative. Any deficiencies or unacceptable work will be noted. Contractor will correct the deficiencies before final acceptance and payment is made.

- SCOPE OF WORK

  - Asphalt Emulsion Sealcoating

    - Prior to sealcoating, all cracks shall be properly sealed with Crafco Polyflex Type 3, product #34521 or equivalent crack sealer. Crack sealer shall be an asphalt-based product designed to be used to fill cracks and joints in asphalt. The product shall have the ability to seal out water.

    - Vegetation growing through the surface of driveways or parking lots that are being prepared for crack sealing and/or seal coating shall be removed and sterilized using a propane torch unit eliminating all vegetation, dirt, moisture and seeds. At the option of the Contractor, a herbicide may be applied prior to the surface treatment application. This shall be applied far enough ahead of the surface treatment application to sufficiently give the vegetation enough time to properly brown so that when completing the final sweeping before surface treatment no vegetation is left. The method of removal is subject to the approval of the County’s Project Manager.

    - If a chemical herbicide is used, it shall be applied according to the manufacturer’s specifications and requires FDACS certification per Chapter 487. The vegetation shall be totally browned before the crack seal and the seal coating is installed. The person applying herbicide shall have, or be under the supervision of someone that has, the proper State of Florida Pesticide Applicators License. A copy of this license shall be supplied to the County’s Project Manager upon request. A log of all herbicides shall be kept, and a copy shall be supplied to the County’s Project Manager. Log shall contain the following information:

      - Proof of licensure for application

      - The type of herbicide

      - The manufacturer of the product

      - The mixture rate used

      - The application rate used

      - The application location

      - The application date and time

      - The weather conditions at the time of the application.

    - Prior to application of coatings, protect adjacent curbs, walks, fences, buildings and other items in the work area.

    - Prior to sealcoating asphalt repairs and crack sealing shall have sufficient curing time as required by the manufacturer’s specifications. The surface shall also be thoroughly cleaned and free from all loose material, dirt, and debris using brooms, air blowers and/or power sweepers. Surface must be dry before coatings are installed.

    - Vegetation removal, crack sealing, and asphalt repair repairs shall be completed as described above.

    - Scrub and clean grease, oil, and gasoline spots with soap and water and prime according to seal coat manufacturer’s specifications.

    - Apply two coats of asphalt emulsion seal coat according to manufacturer’s specifications.

    - Application of the asphalt emulsion seal coat shall be done by using rubber faced squeegees, brooms, distributor bar /wand, or combinations of these or other techniques as approved by the County’s Project Manager.

    - Sealed areas shall be barricaded to traffic and may not be opened to traffic during the curing period as recommended by seal coat manufacturer’s installation instructions.

  - Asphalt Repairs

    - Contractor shall provide on-call asphalt repairs to include base repairs and/or installation, the installation of asphalt, and striping to match the preexisting markings. The repair size will vary. Repairs will take place on County maintained entry roads, parking lots, and trails.

    - Contractor must saw-cut outside the area marked with temporary paint for all asphalt repairs. All saw cuts will be performed to leave only square or rectangular shaped repairs.

    - Contractor must remove existing asphalt, road base material, and any sub-base/sub-grade material necessary to accommodate the installation of a minimum depth of eight (8) inches of compacted lime rock base and one (1) inch of asphalt, unless otherwise directed on the notice to proceed, or to a depth determined by the County Project Manager. Finished elevation of the repair must be at base pre-repair condition and shall allow for the specified depth of asphalt.

    - The area shall be filled with Type S III hot mix asphalt.

    - Cold patch asphalt shall be allowed with approval from County’s Project Manager.

    - No more than one-quarter (¼) inch difference in height shall be allowed for the transition between the patch area and the adjacent area.

  - Asphalt Milling

    - At the discretion of the County, milling may be required to repair an area without performing base repairs.

    - All millings created by this process will become the property of the Contractor and it will be the Contractor’s responsibility to remove them from the project site at no additional cost to the County.

    - County Project Manager will determine the thickness of the area to be milled. Milling will occur in increment of one (1) inch. Contractor will be compensated at the unit price for the milling as specified in the Contractor’s bid.

  - Asphalt Installation

    - For overlays and general base repair, Contractor shall install and compact Type SP 9.5 asphaltic concrete at a minimum thickness of one (1) inch unless otherwise noted. This shall conform to Florida Department of Transportation Standard Specification for Road and Bridge construction, 2019 edition (or latest edition), section 337, Asphaltic concrete friction courses.

    - Asphaltic Concrete Base Course repairs shall utilize SP 12.5 asphaltic concrete at a minimum thickness of one and a half (1.5) inch unless otherwise noted. This shall conform to Florida Department of Transportation Standard Specification for Road and Bridge construction, 2019 edition (or latest edition), section 337, Asphaltic concrete friction courses.

    - For installing new asphalt, the unit cost for tack coat shall be included in the price provided in the pricing sheet.

    - A tack coat shall be applied prior to installation of the new asphalt. The tack coat surface shall be kept free of traffic until the asphalt has been placed.

    - Contractor shall install new asphalt ensuring the transition joints are not excessive.

    - Once the asphalt has been properly compacted, the result of the installation shall provide even transitions to the existing area and have joints that are not excessive, and a good quality ride is provided.

  - Ribbon Curb Installation

    - Installation of 8” x 8” ribbon curb as needed.

    - Excavate dirt as needed to install proper base material.

    - Form ribbon curb utilizing 3,000 PSI, fiber reinforced concrete with broom finish.

    - Control joints should be saw-cut or formed at regular intervals, at 10-foot maximum       centers, to a depth of at least 1/4 of the curb thickness.

  - Striping

Paint striping shall be installed to match existing except that accessible parking spaces shall be installed to meet most current ADA requirements. All work shall be performed in accordance with the requirements of latest edition, at the time the work request is issued to the Contractor, of the following manuals and publications, including, but not limited to: Section 710 from the Florida Department of Transportation “2013 Standard Specifications for Road and Bridge Construction” and any other sections applicable, Florida Department of Transportation “Design Standards” and the Federal Highway Administration “Manual of Uniform Traffic Control Devices.

- COUNTY RESPONSIBILITIES

As stated in Exhibited Sample Contract.

- DELIVERY REQUIREMENTS AND ACCEPTANCE

As stated in Exhibited Sample Contract.

- WARRANTY REQUIREMENTS

As stated in Exhibited Sample Contract.

[End of Exhibit A.]