- BACKGROUND

  - A qualified Engineering Firm shall provide construction, engineering, and inspection (CEI) services for Hartwood Marsh Road from US 27 to Majestic Isle Drive (aka Regency Hills Drive) (FDOT FIN No. 441781-1-58-01) (Project) located in Lake County, Florida. The project is funded with state funds under Florida Department of Transportation (FDOT) Local Agency Program (LAP) FM No. 441781-1-58-01. The engineering firm will also be responsible for all Equal Employment Opportunity Compliance (EOC), including but not limited to reviewing payrolls, conducting Employee interviews, on-The-Job Training form and CUF Reports.  FDOT LAP certification will be desirable but not required.

The Roadway plans shall follow the Florida Department of Transportation, FY 2025-26 Standard Plans for Roadway and Bridge Construction and applicable Interim Revisions (IRs) and July 2025, Standard Specifications for Road and Bridge Construction.

The Standard Plans for Road and Construction and associated IRs can be found at:

https://www.fdot.gov/design/standardplans and

Standard Specifications for Road and Bridge Construction can be found at:

https://www.fdot.gov/programmanagement/implemented/specbooks

- The project consists of Construction, Engineering, and Inspection (CEI) services, materials testing, and post-design engineering services for the project. Consultant shall perform construction oversight including the obligation to assure that all verification testing is performed in accordance with the 2025-26 Standard Specifications for Road and Bridge Construction, as amended.  The project consists of widening Hartwood Marsh Road from US 27 to Majestic Isle Drive (aka Regency Hills Drive), approximately 1.11 miles in length.  The improvements along Hartwood Marsh include widening (to include stabilization, base, asphalt) from a two-lane roadway to a four-lane roadway.  Traffic separators, shoulders, bike lanes, curb and gutter are part of the roadway profile.  The existing roadway will be reconstructed or milled and resurfaced.  A concrete sidewalk is proposed along with the roadway improvements.  Proposed roadway improvements along South Hancock Road include widening (to include stabilization, base, asphalt) from a two-lane roadway to a four-lane roadway.  Traffic separators are proposed for a section of roadway.  The existing roadway will be milled and resurfaced. A concrete sidewalk is proposed along with the roadway improvements.  Gravity walls are proposed throughout the project limits to accommodate for any drop offs. A perimeter wall is also proposed along certain sections of Hartwood Marsh Road.  Drainage will be conducted through a closed drainage system.  Drainage improvements will include installation of storm pipes, inlets, manholes, and mitered end sections.  The stormwater will be directed either into an existing pond or the proposed pond, depending on the drainage basin.  Traffic signals are proposed at the intersections of Hartwood Marsh Road and Fire Station 2, Hartwood Marsh Road and Danbury Mill Boulevard, and Hartwood Marsh Road and Hancock Road.  Traffic signal upgrades are also proposed at Hartwood Marsh Road and US 27.  Signalization improvement include installation of mast arms, open trench and directional bore of conduit, signal cable, fiber optic cable and connection hardware, controller/cabinet, pull boxes, power service and wire, signal priority and pre-emption system, pedestrian signals and poles, vehicular traffic signals, vehicle detection system, pedestrian detector, traffic signal controller assembly and cabinet, ethernet switch, uninterruptible power supply, and remote power management unit.

- DEFINITIONS

The following terms or pronouns used in place of them, will have the following meanings:

  - Addendum – A modification, revision or clarification of this RSQ.

  - BCC Board of County Commissioners - The governing board for Lake County, Florida.

  - Consultant – The Professional Engineer or Engineering Firm registered in the State of Florida who performs Professional Engineering Services for County, other than County personnel. The awarded respondent.

  - County – Lake County, Florida

  - CCNA – Consultants’ Competitive Negotiation Act, Florida Statute 287.055

  - FDOT - Florida Department of Transportation

  - FHWA – Federal Highway Administration

  - LAP - Local Agency Program

  - RSQ – Request for Proposal – A formal solicitation inviting proposals from qualified firms.

  - Response - The information package submitted by qualified firms.

  - Respondent, Consultant, Firm, You and Your - Consultant, person, firm, or corporation who submits a response.

  - Shall, Must or Will – Indicates a mandatory requirement or condition, the material deviation from which will not be waived by County.

  - Should or May - Are permissive in nature. Deviation from such a condition or requirement will not by itself cause automatic rejection of a qualifications package but may be a factor considered in the overall evaluation process.

- QUALIFYING STANDARDS

  - Pursuant to Chapters 471, 472, and/or 481, Florida Statutes, as applicable, Consultant shall be registered with the State of Florida and have obtained at least the minimum thresholds of education and experience required by the applicable statute(s).

  - Consultant must exhibit compliance with the qualification standards and evaluation factors stated in Section 287.055, Florida Statutes. Consultants are advised listing of Subcontractors, and respective percentage of use, are a listed evaluation factor under CCNA.

  - Consultant shall have experience with FDOT documentation of CEI construction projects.

- SCOPE OF WORK

Consultant shall:

  - Provide Construction, Engineering, and Inspection (CEI) services, materials testing, and post-design engineering services for the Projects.

  - Perform construction oversight including the obligation to assure that all verification testing in performed in accordance with the 2025-26 Standard Specifications for Road and Bridge Construction, as amended.

  - Provide inspection services for conformance to Plans and Specifications for all roadway, structures, and specialty items that are being incorporated into the project.

  - Oversee the project’s Contractor to keep the project on schedule.

  - Coordinate the relocation efforts with all utilities in the project area to include new and relocated lines.

  - Observe, measure, and record all quantities for payment.

  - Record field measurements into project records for renew by the County or auditors. The records will be recorded on a standard form (field book) provided by the Consultant and/or on field inspection forms to be submitted to the County.

  - Check traffic control daily, and additionally as required or requested.

  - Notify the Contractor of deficiencies or problems immediately.

  - Document weekly (or as often as necessary) project traffic control on forms provided by the Consultant and distribute as required.

  - Inspect daily erosion control items for conformance to the plans as well as effectiveness in the field.

  - Prepare an accurate daily diary, signed by the inspector, consisting of:

    - A record of the Contractors on the project and their personnel (number and     classification).

    - Equipment on the project (number and type or size) and movement (arriving or leaving the project), along with idle equipment.

    - Location and work performed by each Contractor or Subcontractor.

    - Orders given to the Contractor.

    - Events of note on the project.

    - Accidents on the project and any details surrounding the accident such as police report number, fatalities, causes, time, etc. Obtain a copy of the police report for the project records whenever possible.

    - Weather, amount of precipitation, temperature at morning, noon, and evening, cloudy, clear, etc.

    - Days charged, with explanation if not charged.

    - Any other details that may be important later in the project life.

    - Notify the County’s Project Supervisor prior to withholding payments.

  - Provide effective and qualified supervision of all inspection services being conducted by Consultant and sub-consultants. All field technicians must be certified in the applicable Florida Department of Transportation (FDOT) certification workshops listed below:

    - Asphalt Roadway Paving Inspector

    - Asphalt Plant Inspector

    - ACI Class 1 Concrete Technician

    - Nuclear Gauge Training

  - Provide a Project Supervisor to be always available in case of an emergency on the project and who will maintain a list of Contractor’s personnel that will be responsible for any occurrence that may arise on the project for the life of the project.

  - Schedule, prepare the agenda, attend, and conduct a progress meeting every week with Lake County personnel, Local government entities personnel, Contractor, sub-Contractors, utility personnel, and other agencies affected by the project during construction. The discussion should include recent progress, upcoming events in the schedule, and problems associated with the project. Record significant information revealed and discussed at the meeting and distribute written minutes to the attendees and appropriate agencies.

  - Review and sign-off on all shop drawings and submittals.

  - Provide written responses to the County and Contractor regarding Contractors’ Request For Information (RFI).

  - Be available to respond within thirty (30) minutes in emergency situations or attend a meeting with less than a twenty-four (24) hour notice.

  - Provide all necessary documents to St. Johns River Water Management District (SJRWMD), throughout permitting and construction process.

  - Review and sign-off on the as-builts at project completion and provide certifications to all permitting agencies included SJRWMD and National Pollutant Discharge Elimination Systems (NPDES) permitting.

  - Maintain an open format communication with the Local government entities and their utility Contractors.

  - Provide surveying services as needed.

  - Provide subsurface Utility Engineering as needed.

  - Provide continuous monitoring and enforcement of Maintenance of Traffic (MOT) plans (with at least one employee has completed the Advanced Maintenance of Traffic Level Training).

  - Provide continuous monitoring and enforcement of the NPDES permit requirements.

  - Prepare for and attend, when requested, any periodic or in-depth County inspection that may be conducted on the project related to project work, progress, or records.

  - Prepare for, cooperate with, and assist auditors that may be assigned to review project records, payments, reports, etc.

  - Monitor Consultant hours worked on the project and justify need for overtime. Prior to starting work, submit to the County Project Supervisor a listing of personnel assigned to the project for revie and approval. In addition, a list of persons with emergency phone numbers should always be supplied to the County and Contractor.

  - Provide Quality Assurance, Testing for Acceptance, and Training. Monitor the testing provided by the Contractor in the field as defined in the Contract, Plans or Specifications. Document Consultant testing on standard FDOT type forms and distribute as required. Monitor documentation of testing by the Contractor. Field testing by the Consultant includes, but is not limited to, all ACI tests for concrete including concrete plant for acceptance by the County, nuclear density testing of earthwork, base stone, asphalt, structural backfill, and pipe backfill as defined in the Standard Specifications and the County’s sampling and testing schedule attached as Exhibit A. The Consultant will also provide aggregate analysis and moisture testing for roadway embankment and base stone materials as defined in the Standard Specifications and the County’s sampling and testing schedule. Also included as the responsibility of the consultant is miscellaneous checking of application rates and dimensions and bearings to assure conformance to Plans and Specifications. In case of notification of defective concrete, asphalt or other product as defined in the Specifications, the Consultant will submit the initial information on forms provided by the Consultant and receive the final disposition of the material after review. Certifications of material submitted by the Contractor will be reviewed by the Consultant for conformity to the Specifications. The certification documents submitted to the County will also be reviewed for completeness and conformance to the County’s standard form of submission. A Final Materials and Tests Certification will be submitted to the Materials and Tests Manager with the Final Records.

  - Review Progress Payments: The Consultant shall review and document accurate quantities for Monthly Progress Payments from the prime Contractor based on actual project field records. The quantities for payment shall be referenced to field records prior to submission to the County for payment. Test reports will be on file prior to payment. The County Project Supervisor must approve any waiver of testing documents prior to payment. Pay quantities will be submitted to the County Project Supervisor for review and payment.

  - Review Revisions to the Contract Plans: Anu revisions to the Contract plans or cross section shall be submitted to the County Project Supervisor for processing.

  - Submit to the County Project Supervisor a copy of all correspondence between the Consultant, Contractor, Subcontractors, or others concerning matters related to the project and maintain an office file copy of submission with the project Final Records.

  - Submit a compilation of Final Project records in the FDOT standard format to the County after project completion. Make corrections when/if notified and resubmit the records and the consultant’s final pay estimate for the project at the appropriate time. Submit all final forms with the final records. Coordinate Consultant hours after the project completion with the County Project Supervisor for approval.

  - Prepare documentation and assist in the defense of the County, when requested, in preparation for claims or possible claims resulting in the execution of the contract.

  - Provide designated materials testing laboratory by a certified FDOT Laboratory for all applicable testing requirements (See Section 5.0). All materials must meet FDOT specifications. Monitoring the testing to ensure all requirements are met.

  - Communicate on all levels of project personnel, from field inspectors striving to resolve issues at the lowest level possible and escalating, if necessary, to senior management staff. Phone and email will be the main forms of communication on a daily basis. The Consultant will distribute meeting minutes after each project progress meeting and pre-activities meetings. Lake County and firm will conduct weekly staff meetings for the project communication and utilize email and conference calls, as needed. Communication with the Contractor will take the form of weekly project progress meetings, pre-activity meetings, phone calls, e-mails, and face-to-face discussions.

  - Ensure the Contractor’s CPM schedule is developed, implemented, and updated per the contract. Maximize MOT by coordinating with any active construction operations potentially impacting the project. Develop and review the emergency response plan during the construction phase and coordinating with the County’s plan.

  - On-site Inspection: CEI Contractor will assist the COUNTY as needed to monitor the Road Contractor's significant on-site construction activities and inspect materials entering into the work in accordance with the plans, specifications, and special provisions for the Construction Contract to determine that the project is constructed in reasonable conformity with such documents.

  - The COUNTY shall keep detailed accurate records of the Contractor's daily operations and of significant events that affect the work. The COUNTY shall be responsible for monitoring and inspection of Contractor's Work Zone Traffic Control Plan and review of modifications to the Work Zone Traffic Control Plan, including Alternate Work Zone Traffic Control Plan, in accordance with COUNTY's procedures. Contractor may assist the COUNTY and employees performing such services shall be qualified in accordance with the Department's procedure.

  - Sampling and Testing: Contractor shall perform verification sampling and testing of component materials and completed work in accordance with the Construction Contract documents. The minimum sampling frequencies set out in the Department's Materials Sampling, Testing and Reporting Guide and/or The COUNTY specifications shall be met. While onsite Contractor shall provide daily surveillance of the Contractor's Quality Control activities at the project site and perform the sampling and testing of materials and completed work items that are normally done in the vicinity of the project for verification. Contractor shall be specifically responsible for job control samples determining the acceptability of all materials and completed work items on the basis of either test results or verification of a certification, certified mill analysis, FDOT label, FDOT stamp, etc. Sampling, testing and laboratory methods shall be as required by the COUNTY's Standard Specifications, Supplemental Specifications or as modified by the Special Provisions of the Construction Contract. Documentation reports on sampling and testing performed by Contractor shall be submitted to responsible parties during the same week that the construction work is done, Contractor will furnish all acceptance testing information and data to the COUNTY in an acceptable format.

  - Engineering Services: Engineering services include maintaining the required level of surveillance of Contractor activities, interpreting plans, specifications, and special provisions of the Construction Contract, maintaining complete, accurate records of all activities and events relating to the project, and properly documenting all significant project changes. Contractor shall perform; but, not be limited to the following services:

  - Assist the COUNTY with the preparation of the pre-construction meeting(s) and attend pre- construction meeting(s) with the Contractor and COUNTY.

  - Coordinate the relocation efforts with all utilities located in the project area. Schedule and take the lead on bi-weekly progress meetings during construction.

  - Provide written responses to the COUNTY regarding contractors’ requests for information (RFI).

  - Make site visits to answer field questions or to make necessary field adjustments. Review and sign off on the as-builts at project completion.

  - Provide close out documents to satisfy FDOT Local Agency Agreement. Clear applicable water management permits.

- LABORATORY TESTING AND SAMPLING SCHEDULE

| OPERATION | MATERIAL SPECIFICATION | TESTS | PROJECT REQUIREMENTS | TESTING FREQUENCY |
| --- | --- | --- | --- | --- |
| Prime and Tack Coats | FDOT Standard Specifications (Latest Edition) |  | Certification | Every Transport |
| Type Superpave Asphaltic Concrete | FDOT Standard Specifications (Latest Edition) | Job Mix Formula | Certification | Each mix design or change of aggregates |
|  |  | RICE (Gmm) | Section 334 | One per sublot |
|  |  | Extraction Gradation Analysis | Section 334 | One per sublot |
|  |  | Field Density (Gmb) | Section 334 | 5 six-inch cores per sublot (Random Locations) |
|  |  | Asphalt Binder | Section 334 | 1 sample for the first 1000 tons and 1 per 4000 tons after |
|  |  | Thickness | Section 330 | Daily |
| Pavement Smoothness | FDOT Standard Specifications (Latest Edition) | Rolling Straight Edge | Section 330 (FM 5- 509) Max 3/16 inch | Final SP structural layer and friction course layer |
| Concrete | FDOT Standard Specifications (Latest Edition) | Compression Strength | FDOT Section 346 and 347 | Section 347 acceptance based on Certification; Section 346 One (1) set of cylinders for 10 CY or more per day. Additional set(s) for each 50CY/day. One(1) set for each class of concrete placed each day. |
|  |  | Each class of concrete used | Certification | Each mix design or change |
| Embankment | FDOT Standard Specifications (Latest Edition) | Standard Proctor AASHTO T-99 | Section 120 | Per material type |
|  |  | Field Density | Section 120 -100% AASHTO T-99180 | Section 120-One test per 500 LF per 12” lift of embankment Section 125-One test per 300 LF of pipe trench (or between structures) per 6’ lift until 1 ft above pipe; 12” lift of backfill 1 ft above pipe Section 125 (modified) One test per 12” lift of structure backfill alternating sides |
| Compacted Subgrade | FDOT Standard Specifications (Latest Edition) | Standard Proctor AASHTO T-99 | Section 120 | One per material type |
|  |  | Field Density | 100% AASHTO T- 99 | Section 120 (modified)-One per 300 LF of sidewalk One per driveway |
| Stabilized Subgrade | FDOT Standard Specifications (Latest Edition) | Limerock Bearing Ratio FM 5-515 | Section 160 | One per 1,000 LF per lane (One per 2 lots) One per 2,000 LF per shoulder (One per 4 lots) |
|  |  | Field Density/Thickness | Section 160-98% AASHTO T-180 | Density-One per 500 LF per lane (1 per lot) Thickness – 3 per lot Density/thickness (modified)-One per 500 LF per shoulder |
| Limerock Base Course | FDOT Standard Specifications (Latest Edition) | Modified Proctor AASHTO T-180 | Section 200 | One per 4 8 lots |
|  |  | Field Density/Thickness | Section 200-98% AASHTO T-180 | Density-One per 300 LF per lane Thickness- 3 per lot Density/Thickness-One per 500 LF / shoulder |
| Sodding | FDOT Standard Specifications (Latest Edition) Section 575, Sodding, and Section 981 | Each type of sod used | Certification | All seed, sod, and mulch shall be free of noxious weeds and exotic pest plants, plant parts, or seed listed in the current Category I “List of Invasive Species” from the Florida Exotic Pest Plant Council |

- PROJECT LOCATION

Hancock Road

Regency Hills Rad

Hartwood Marsh Road

US 27