Federally-Funded Grant Award #: SLT-3284

This project is funded through the American Rescue Plan Act (ARPA), Coronavirus State & Local Fiscal Recovery Funds (SLFRF).  Contractor and subcontractors shall comply with the terms of the Grant Agreement.

- CONTRACTOR RESPONSIBILITIES

Contractor shall:

  - Be licensed and fully competent in all aspects of land development and residential construction in a safe manner.

    - Employ only skilled, qualified workers.

    - Be responsible for any needed maintenance of traffic.

  - Provide all-inclusive quotes to provide 100% turnkey land development services and construction of twenty-one (21) residential dwelling units for the proposed County’s Residential Village development located at 28123 County Road 561, Tavares, FL 32778, as described in the Multi-Family Detached Housing Lake County Construction Plans, prepared by Halff, for Lake County, including all off-site work required for connection of the development to city water and sewer services.

    - Include all required labor, materials, equipment, manufacturer’s installation documentation, components/devices, transportation, fuel, supervision, surveying, permits, inspections, and all other incidentals necessary to complete all necessary work in accordance with all parts of this solicitation and bid documents provided.

    - Include costs for general housekeeping and work area clean up.

    - Include travel time.

    - Change orders shall not be issued for incidental items or tasks that should have been reasonably construed to be part of the project by the Contractor.

  - Provide all plans, engineering, and other requirements for eighteen (18) single-bedroom residential dwelling units and three (3) two-bedroom ADA compliant residential dwelling units, including 100% turnkey installation of the residential dwelling units, in ready-to-lease condition, per the minimum requirements set forth herein. Design to include concrete slab and footing to support proposed dwelling units structure.

  - Complete all work as specified in signed and sealed construction documents.

  - Obtain licenses, permits, and fees (including inspection fees) as required to comply with all laws, ordinances, regulations, and code requirements applicable to complete the project.

  - Be responsible for inspections, penalties, fees, or fines for project.

  - Enforce safety regulations and maintain a clean and safe work environment.

  - Be responsible for damages caused as the result of completing project.

  - Furnish all tools and equipment, including, but not limited to, site clearing and grading equipment, cranes, lift trucks, boom trucks, cherry pickers, or any other tool or equipment needed to complete the project timely.

  - Provide and maintain sanitary facilities, dumpsters, trash cans, storage containers, and any other equipment needed during construction.

  - Be responsible for scheduling, leading, and documenting project progress meetings with the architect of record and county representatives.

  - Attend any additional meetings scheduled by the county representatives to review topics related to the Project.

  - Create and maintain an up to-date project construction schedule.

  - Document activities, prepare, and distribute construction daily reports.

  - Produce and maintain to-date submittal, and Request for Information (RFI) logs.

  - Produce a valid schedule of values prior to project commencement.

  - Produce punch lists and address punch items in a timely manner as required by the county representative.

  - Document all changes and product information needed to file drawing revisions (red-line drawings).

  - Obtain all closeout permits, certificates of occupancy, and documentation needed as requested by the owner.

- SCOPE OF WORK:

  - Contractor is responsible for locating/identifying all utilities before starting construction; refer to Utilities section below for additional requirements. Contractor must take every precaution to avoid damage to any underground utilities. Contractor will be responsible for any damage caused to the existing utilities. In the event that Contractor or any of its subcontractors damage an existing utility, the Contractor will be responsible for the repair and must repair the same at its own cost.

  - All existing utilities (water/sewer, electrical, telephone, cable, etc.) service connections, interruptions, or relocations shall be coordinated by Contractor in advance with field representatives of each respective utility.

  - No tree burning allowed. Contractor will be responsible for removing and disposing of existing trees in a legal and proper manner as necessary for the construction of the proposed improvements.

  - Contractor shall be responsible for providing all necessary clean fill material, grading, and compaction to level site area before construction of proposed improvements.

  - The Contractor will be responsible for planning and providing perimeter site fencing and signage to ensure the safety and security of the site from the public.

  - Contractor is responsible for the installation of silt fencing for entire perimeter of working area prior to starting any construction activity and stabilized construction access.  Contractor shall be responsible for installing inlet protection as required and maintaining all erosion and sediment control BMPs for the duration of the project.

  - Contractor must provide compaction test results demonstrating that subgrade has been compacted to 98% AASHTO T-180 prior to pouring concrete and construction of asphalt driveway and parking.

  - Contractor must obtain Lake County Facilities Management staff inspection approval prior to pouring concrete. Contractors shall submit the Concrete Pad, Sidewalk & Ribbon Curb Pre-Pour Inspection/Approval Form, included as part of the bid documents, to the Office of Parks and Water Resources, Facilities Management for approval a minimum two (2) days before pouring concrete. This inspection is in addition to inspection by the Building Inspector.

  - All concrete sidewalks shall be a minimum of 6” thick (not 5 ½” or 5 ¾”). Wire mesh shall be mounted on chairs.

  - Contractor is responsible for debris/trash management and must provide a dumpster or remove all construction debris/discarded materials/trash daily. No discarded construction material shall be dumped or remain on site unless contained within a dumpster provided by the Contractor. Dumpsters shall be maintained to prevent overflow of debris.

  - Contractor responsible for re-grading and re-sodding all areas impacted by construction of the proposed improvements. Contractor shall provide all sod required. No grass seeding allowed.

  - Lake County Parks and Water Resources, Facilities Management will be providing the Building Permit (Plan Review Only) for the proposed improvements, however, the Office of Building Services has indicated Contractor will be required to complete a Building Permit Application prior to issuance of this Permit. In addition, the Contractor and any subcontractors must be added to the building permit and pay all related permit and inspection cost/fees for the required work included under this solicitation. Refer to Permit Requirements section below for additional requirements.

  - Contractor will be required to provide a timeline/work schedule and a detailed cost breakdown (schedule of values) at the preconstruction meeting for review which shall be the base for all payment applications.

  - Contractor shall provide 100% turn-key construction of eighteen (18) single-bedroom residential dwelling units and three (3) two-bedroom, ADA compliant residential dwelling units, including all engineering, permitting, and other requirements for the project, which shall meet the minimum requirements specified herein.

  - Dwelling Units Minimum Requirements:

    - Units provided shall fit within the building envelopes provided in the site plans and should have the following minimum square footage:

      - Single-bedroom (1 bedroom/1 bath) – 600 sq-ft

      - Two-bedroom (2 bedroom /1 bath) (ADA compliant) – 700 sq-ft

    - Bedrooms shall include a closet, ceiling fan, and smoke & carbon monoxide detector.

    - Bathrooms shall include at minimum a toilet, bathroom vanity with sink, mirror, shower, towel rack, toilet-paper holder, and exhaust fan vented to the exterior. Shower walls and floor shall be provided with Porcelain Tile, color to be selected by Lake County staff.

    - Kitchen shall include a kitchen sink with garbage disposal, kitchen hood with vent to the exterior, range 240-volt electrical connection, range, water valve connection for refrigerator, refrigerator with external water/ice dispenser, dishwasher, and kitchen cabinets with solid surface countertop (color/finish to be selected by Lake County staff).

    - Living Room with ceiling fan. Cable TV and internet connections shall be provided.

    - Laundry Room with washer/dryer combo, including washing machine outlet box with hot/cold water/hammer arrestors and drain connections, 120-volt and 240-volt electrical connections.

    - Flooring: All areas shall be provided with waterproof laminate flooring except for the bathroom which shall be provided with Porcelain Tile. Color to be selected by Lake County staff. Waterproof laminate flooring shall be installed over moisture barrier underlayment.

    - Lighting system is required for all interior areas and exterior front area of the dwelling units. All interior and exterior lighting fixtures shall be LED.

    - Water heater.

    - Electrical systems include all necessary electrical components but not limited to electrical meter and panel, main and circuit breakers, electrical outlets, light switches, conduits and wiring.

    - Central heating, ventilation, and air conditioning systems (HVAC) system with digital programable thermostat.

    - Ceiling height minimum is eight (8) feet.

    - NFPA 13D compliant residential fire sprinkler system.

    - Roofing: Contractor to furnish and install all roofing materials including accessories and incidental work required to provide a complete and watertight roofing system. Roofing must be installed in strict accordance with the manufacturer's specification and all applicable Standards. Roofing shall be certified to be manufactured and installed for compliance with the Florida Building Code.

Roof Color: Sierra Tan (Standard Color).

Vapor Barrier: Peel and stick self-sealing and self-adhering membrane minimum 40 mil thick.

Warranty: Contractor shall provide a manufacturer's written full non-prorated warranty, covering roofing, flashing and trim, all of which shall be products of the roofing manufacturer. The warranty coverage shall include all flashings, penetrations, underlayment, field applied gutter material, and edge details as part of the weathertightness warranty. Contractor shall also provide a workmanship guarantee, on the roofing installation and components.

    - Modular Building Manufacturer: Exterior and interior walls including roof framing may be all steel framing by a Certified Modular Manufacturer.

    - Dwelling including unit design/construction shall be in compliance with the Florida Building Code latest edition and all applicable local and Federal codes.

Contractor shall provide and submit engineering drawings per above dwelling unit requirements for review/approval to the Office of Parks and Water Resources, Facilities Management before proceeding to apply for building permits. Dwelling unit drawings must be signed/sealed by a professional engineer.

  - Construction includes but is not limited to; permitting, site preparation; clearing and grubbing; demolition and removal of the existing asphalt driveway aprons, removal of existing chain-link fence and PVC fence (east and south side of property only), grading; construction of temporary gravel construction entrance; construction of stormwater system, including installation of concrete storm drain piping and mitered end sections (MES), inlets, flared end sections (FES), swales, yard drains, weir and retention ponds; construction of asphalt driveway and parking; installation of signs; construction of concrete curbs; installation of ADA detectable warning mats; pavement markings (thermoplastic), construction of dumpster enclosure, bus shelter, mail kiosk, bike racks, installation of vinyl fence, and installation of silt fencing.

  - Contractor shall be responsible for National Pollutant Discharge Elimination System (NPDES) permit compliance, including preparation of Stormwater Pollution Prevention Plans (SWPPP), site inspection and documentation, and installation and maintenance of appropriate Best Management Practices (BMPs), including silt fence, inlet protection, etc.

  - Water Service: Consisting in the connection to the existing watermain, master meter with backflow preventer device, fire hydrants, water main and water service lines, gate valves, including all necessary components to provide water service to the proposed dwelling units under this bid.

  - Sewer: Installation of offsite force main and connection to existing City of Tavares offsite lift station, construction of a lift station including fencing/gate and connection to the proposed sanitary sewer system, installation of sanitary manholes, sanitary sewer lines, cleanouts, including all necessary components to provide sanitary sewer service to the proposed dwelling units under this bid. Installation of force main crossing County Road 561 by directional boring only.

  - Electrical			 Service: Contractor to furnish and install a turn-key electrical system including but not limited to installation of transformer pads, distribution panels, installation of electrical conduits, conductors and electrical panels including all necessary electrical components to provide power service to proposed dwelling units, lift station, and site lighting.

  - SECO Energy: Removal of existing lighting poles. Contractor responsible for coordinating all work with SECO Energy.

  - Final Payment: A Certificate of Occupancy/Completion issued by the Office Building Services, (352) 343-9653, must be provided on or before submittal of the final payment application.

- ACCEPTANCE

Lake County will review all work to determine compliance prior to final acceptance of the work. Any work not installed in accordance with the plans and specifications and rejected by Lake County must be removed and replaced at the Contractor’s expense.

- CLEANING

Perform cleaning during installation of the work and upon completion of the work. Remove from site all excess materials, soil, debris, and equipment. Any spoils created from this work must become the property of the Contractor and must be disposed of in a legal and proper manner.

- FIELD CONDITIONS

It is the responsibility of the Contractor to verify all site conditions before they submit their bid. Verify and coordinate all work to field locations and dimensions. Contractor is responsible for the installation of temporary construction fencing and signage to safely enclose the work area along with silt fencing for entire perimeter of site work.

- INSTALLATION

Installation must be in accordance with the terms and conditions in this Invitation to Bid, construction documents provided and as stated in the Scope of Work.

All work must meet all applicable Federal, State, and local building codes and must be performed by qualified licensed contractors in their respective areas (electrical, mechanical, plumbing, fire, etc.). The Contractor must complete all work per the Construction Documents, including the associated specifications.

- LEAD TIME

Any long lead item must be ordered within one (1) week of issuance of project purchase order. Proof of purchase will be required.

- PERMIT REQUIREMENTS

Contractor is responsible for obtaining all necessary building permits and inspection approvals and paying all related costs/fees for the work included under this solicitation. Contractor is responsible for obtaining directly from the engineering firm at his own expense, if needed, any necessary engineering drawings (CAD drawings, signed/sealed electronic plans or hard copies). Contractor must provide copies of all permits obtained by Contractor to Parks and Water Resources, Facilities Management prior to the start of construction. Contractor responsible for installing a Permit Posting Box Unit. Approved plans and permits must be available on site for the inspector. For questions regarding building permits and review process please contact the Office Building Services at (352) 343-9653.

- PRODUCT HANDLING

Contractor is responsible for the proper packaging, shipping, handling and storage of materials to be incorporated in the work, so as to ensure the preservation of the quality and fitness of the materials for proper installation as required by the contract documents. Products are to be delivered to the site in the manufacturer’s container or packaging. Contractor shall provide the following: storage containers for material as necessary; sufficient manpower and equipment necessary to off load equipment/materials at jobsite per scheduled deliveries; and dumpsters for cardboard waste and packing debris. Contractor must store materials in a secure, weather-protected area and provide adequate security to protect delivered and stored products from theft, vandalism, or damage during the installation. Contractor is responsible for the return of all damaged products to manufacturer and the replacement of damaged items.

- PROPERTY REPLACEMENT

Property at the site, including turf (rubber, sod), concrete pavement, asphalt, lime rock path, and fencing removed or relocated during the course of construction and any damage due to the work performed by Contractor, must be reinstalled, repaired or replaced before final payment is made.

- SAFETY PRECAUTIONS

Contractor shall be responsible for initiating, maintaining, and supervising all safety-related precautions in connection with the work throughout the contract term. Contractor must erect and maintain all reasonable safeguards for safety and protection, including posting danger signs and other warning signs against hazards. It is the responsibility of the Contractor to provide a safe environment for Contractor’s employees, subcontractors, and all other users, including County staff, and must monitor and maintain fencing/barricades to ensure continued safety.

- USE OF SITE

The Contractor must confine its operations at the site to areas permitted by law, ordinances, and permits. The Contractor must not unreasonably encumber the site with materials, equipment, or trailers. The Contractor shall not cause the site entrance to be blocked or otherwise prevent reasonable access to the site, other working and parking areas, completed portions of the work and/or properties, or storage areas. The Contractor will be given 24-hour notice by the County to remove or relocate any materials, equipment, or trailers improperly placed on the site. If Contractor fails to remove or relocate the items within the 24-hours provided, the County will remove the items at the Contractor's expense.

- UTILITIES

Contractor is responsible for making appropriate contact to ensure utility locations have been marked prior to excavation and trenching. Call 811 before you dig (www.Sunshine811.com). The Contractor must protect all utilities (including irrigation) encountered while performing its work. The Contractor must exercise due care when excavating around utilities and must restore any damaged utilities, at its own expense, to the same condition or better as existed prior to start of work.

- VENDOR SUBMITTAL

Submit product data/specifications and shop drawings electronically to Parks and Water Resources, Facilities Management for review and approval for all items of work. All submittals are to be approved by the engineer/architect and/or Parks and Water Resources, Facilities Management prior to delivering materials to job site and installation.

- AS-BUILT DRAWINGS

One (1) complete set of As-Built Drawings (signed/sealed by a licensed professional) in hard copy and electronic format along with operation and maintenance manuals (if applicable) shall be supplied to the County upon completion of the work and before submitting final payment application.

- WARRANTY AND GUARANTEES

Contractor must furnish Parks and Water Resources, Facilities Management with a signed minimum one (1) year warranty and maintenance program covering all systems from the date of final acceptance of the project by Lake County. The Contractor is responsible for securing warranties and guarantees for any materials, equipment, or fixtures to be incorporated into the project.

- WORKMANSHIP

  - All furnishings described in this solicitation must be installed by qualified tradesman. All installation work and materials to be per manufacturer’s specifications, or as directed by the County’s Representative.

  - All work and materials are subject to the approval of the County.

  - All products must be inspected by the County for damage and chipped or marred finish.  Contractor must replace any damaged or rejected products at no additional cost to the County. The County’s representative may at the County’s option authorize acceptance of chipped or scratched painted surfaces repaired by Contractor at his own risk.

- LIQUIDATED DAMAGES

  - Contractor understands that due to the federal funding source, time is of the essence in the completion of the work described herein.

  - Substantial Completion: The date of substantial completion of work or designated portion thereof is the actual date certified by the County (Owner) or designated authorized representative, when the construction services are sufficiently complete, in accordance with all stated terms and conditions set forth in the solicitation. The County’s authorized representative may authorize the release of the following upon receipt and acceptance.

The date of substantial completion may include but is not limited to the following:

- Approval and Receipt of Occupancy Permits by Fire Marshall;

- All materials and equipment installed;

- Landscaping and site work complete;

- HVAC test and balance complete and all certificates submitted to County;

- All systems in-place, functional and displayed to County’s authorized representative or designee. All systems signed off by the County or County’s authorized representative or designee;

- Cleaning and removal of debris from premises;

- Receipt of manuals and/or warranty certificates/information by the County;

- Draft record documents submitted to County/or authorized representative or designee; and

- Training and on-site demonstrations complete.

  - Final Completion: County and Contractor agree time is the essence for the completion of this project. The Contract time shall begin with issuance of a Notice to Proceed (NTP) indicating the start date to the Contractor by the County. Final Completion shall be 30 calendar days after Substantial Completion.

The date of final completion may include but is not limited to the following:

- Work is complete and in accordance with the Contract Documents and is ready for final inspection.

- Completion and approved Operation and Maintenance manuals have been submitted.

- Record Drawings reflecting “as built” conditions are complete and acceptable to the Architect-Engineer.

- All required Project Close-out Documentation is complete.

- Full Occupancy Permit from the Department for Housing, Buildings and Construction has been obtained.

All additional materials required by the specifications have been delivered to the Using Agency. E.g., spare parts, ceiling tiles, floor title, etc.

  - Liquidated Damages:  Time is of the essence in the work provided for in these construction documents and there will be, on the part of the County, considerable monetary damage in the event the work is not completed within the time fixed for the completion of the contract, or within the time to which such completion may be extended by consent of the County.  Inasmuch as the actual damages for such delay is impossible to exactly determine, the bidder agrees that he/she and his/her surety shall be liable for and shall pay to the County the following liquidated damages:

For failure of the Contractor to achieve final completion of the Work within the time set forth in the contract between County and contractor; liquidated damages shall be in accordance with the following schedule:

| CONTRACT AMOUNT | LIQUIDATED DAMAGES PER CALENDAR DAY |
| --- | --- |
| $10,000 to $100,000 | $100 |
| $100,000 to $499,999 | $250 + 0.05% of contract amount exceeding $100,000 |
| $500,000 to $999,999 | $450 + 0.04% of contract amount exceeding $500,000 |
| $1,000,000 to $1,999,999 | $650 + 0.03% of contract amount exceeding $1,000,000 |
| $2,000,000 to $5,999,999 | $950 + 0.02% of contract amount exceeding $2,000,000 |
| $6,000,000 and Greater | $1,750 + 0.01% of contract amount exceeding $6,000,000 |

  - For the purpose of computing liquidated damages only, no deduction shall be made from the Contract amount for County Direct/Contractor Assisted Purchase of Project Related Material and Equipment. If the project has multiple phased substantial completion dates, liquidated damages for each phase shall be computed based on the contract amount allotted to each phase by the schedule of values.

- EQUAL” PRODUCT CAN BE CONSIDERED UPON RECEIPT OF SPECIFIED DATA

  - Vendors must submit any equivalent request two (2) weeks prior to bid opening. Failure to submit timely will make equivalent submittal non-responsive.

  - The manufacturer's name, brand name and/or model number information contained in this solicitation are being used for the sole purpose of establishing the minimum requirement of level of quality, standard of performance, and design and is in no way intended to prohibit the offer of another manufacturer's items of equal material unless otherwise indicated elsewhere in this solicitation.

  - This specific solicitation requires submission of the following documentation to enable County evaluation of “equal” products:

______:  Product Information Sheets

______:  Product Samples with Initial Offer

______:  Product Samples Upon Specific Request

______:  Product labels

______:  Performance Test Results

  - If an “equal” product may be considered by the County in accordance with this solicitation, the unit shall be equal in quality and standards of performance to the item specified in the solicitation. Where an “or equal” item is offered, and product information sheets are required, the initial offer must be accompanied with a complete set of product information sheets (such as factory specifications, standard manufacturer information sheets, catalogues, and brochures), and if required, a copy of performance test results of the unit offered as an equal. For product information submittals, all supporting documentation submitted by the vendor must in total meet the required specifications set forth in this solicitation. Where the standard product literature submitted with the offer provides information that does not comply with the specifications, the vendor shall state, in an official letter on corporate letterhead as part of their initial offer, the differences between the item they are specifically offering, and the equipment described by the standard product literature, to substantiate compliance to all of the specifications set forth in this solicitation. In such cases, any offer submitted with standard product literature but without the letter explaining compliance will result in the rejection of the offer for not meeting the solicitation specifications.

  - If samples of all “or equal” items bid are required for evaluation, such items are to be provided at no cost to the County, and should be submitted with the initial offer, or at the time of specific request. Failure to meet this requirement may result in your offer being rejected.

  - For “equal” products to be evaluated based on submission of product labels, the initial offer should be accompanied with labels indicating the specification and ingredients for each “or equal” item offered. Failure to meet this requirement may result in your offer being rejected.

  - The County shall be sole judge of equivalence, based on the best interests of the County, and its decision in this regard shall be final. Items labeled "No Substitute" on the County’s Bid/Proposal Submission Form are the only products that will be accepted under this solicitation.

[End of Exhibit A.]