- SCOPE OF WORK:

  - A “qualified licensed contractor” (hereinafter “Contractor”) shall furnish all labor, materials, equipment, component/devices, transportation, fuel, supervision, surveying, permits, inspections, and all other incidentals necessary to complete all necessary work listed, all in accordance with all parts of this solicitation and bid documents provided for the property located at Village Market Place, 365 E. Burleigh Boulevard, Tavares, FL 32778.

  - Contractor is to provide complete, turn-key for the Supervisor of Elections Building Tenant Improvements as described in the Supervisor of Elections Building TI Construction Drawings, prepared by Forefront Architecture and Engineering, for Lake County.

  - Building Tenant Improvements include but are not limited to:

    - Demolition: Removal and disposal of existing materials (including but not limited to: walls and ceiling, electrical components including lighting, switches, receptacles and associated conduits, cables, boxes, supports and devices, doors/frames, windows, flooring; restroom toilets, sinks and accessories; roll-up door, mezzanine metal ladder, mechanical vents).

    - Construction of new footings and steel columns, construction of new frame walls with metal studs, installation of storefront and interior windows/doors including hardware, lintels, operable partitions, construction of bullet resistant wall, installation of cabinets, installation of flooring, new stair for mezzanine, construction of vehicular barrier, installation of signs; installation of new loading dock bumpers.

    - Plumbing System: Consisting in the furnishing and installation of plumbing fixtures, electric tankless water heaters, construction of new shower, including all necessary components and connection to the existing water/sewer service.

    - Electrical Service: Contractor to furnish and install a turn-key electrical system including but not limited to installation of emergency power generator system (including connection to existing natural gas line), switchboard electrical panels, distribution panels, installation of electrical conduits, conductors, lighting system, switches, receptacles, electrical hand dryers for the restrooms, including all necessary electrical components to provide power service. Contractor to also provide a turn-key Extra Low Voltage (ELV) Technology/Communications System including Voice/Data Cable Infrastructure System and all necessary components.

SECO Energy: Contractor responsible for coordinating all work with SECO Energy.

TECO Energy (Gas Company): Contractor responsible for coordinating all work with TECO Energy for gas service connection to the emergency power generator.

    - Fire Sprinkler and Alarm System: Contractor to connect proposed fire sprinkler system to the existing fire main and provide all necessary devices, modules, programming, testing, and will be also responsible to tie tenant space into the base-building fire alarm system. Contractor shall provide engineering signed/sealed drawings and obtain required permit for a complete code-compliant system.

    - Mechanical System: Consisting of turn-key Heating, Ventilation and Air Conditioning (HVAC) system including but not limited to the installation of dehumidification units, Variable Refrigerant Flow (VRF) indoor units, smoke detector sensors, exhaust fans, digital programmable thermostat/humidistat, condensate drains, including all necessary mechanical components to provide functional HVAC system.

- CONTRACTOR RESPONSIBILITIES:

  - Contractor is responsible for locating/identifying all utilities before starting construction; refer to Utilities section below for additional requirements. Contractor must take every precaution to avoid damage to any underground utilities. Contractor will be responsible for any damage caused to the existing utilities. In the event that Contractor or any of its subcontractors damage an existing utility, the Contractor will be responsible for the repair and must repair the same at its own cost.

  - All existing utilities (water/sewer, electrical, telephone, cable, etc.) service connections, interruptions, or relocations shall be coordinated by Contractor in advance with field representatives of each respective utility.

  - The Contractor will be responsible for planning and providing perimeter site fencing and signage to ensure the safety and security of the site from the public.

  - Contractor is responsible for debris/trash management and must provide a dumpster or remove all construction debris/discarded materials/trash daily. No discarded construction material shall be dumped or remain on site unless contained within a dumpster provided by the Contractor. Dumpsters shall be maintained to prevent overflow of debris.

  - Lake County Parks and Water Resources, Facilities Management will be providing the Building Permit (Plan Review Only) for the proposed improvements, however, the Office of Building Services has indicated Contractor will be required to complete a Building Permit Application prior to issuance of this Permit. In addition, the Contractor and any subcontractors must be added to the building permit and pay all related permit and inspection cost/fees for the required work included under this solicitation. Refer to Permit Requirements section below for additional requirements.

  - Contractor must provide a detailed cost breakdown (schedule of values) using Pricing Sheet attached to this bid, which shall be the base for all payment applications.

  - Contractor shall also provide a timeline/work schedule at the preconstruction meeting for review.

  - Final Payment: A Certificate of Occupancy/Completion issued by the Office Building Services, (352) 343-9653, must be provided on or before submittal of the final payment application.

- ACCEPTANCE

Lake County will review each installation to determine compliance prior to final acceptance of the work. Any work not installed in accordance with the plans and specifications and rejected by Lake County must be removed and replaced at the Contractor’s expense.

- CLEANING

Perform cleaning during installation of the work and upon completion of the work. Remove from site all excess materials, soil, debris, and equipment. Any spoils created from this work must become the property of the Contractor and must be disposed of in a legal and proper manner.

- FIELD CONDITIONS

It is the responsibility of the Contractor to verify all site conditions before they submit their bid. Verify and coordinate all work to field locations and dimensions. Contractor is responsible for the installation of temporary construction fencing and signage to safely enclose the work area along with silt fencing for entire perimeter of site work.

- INSTALLATION

Installation must be in accordance with the terms and conditions in this Invitation to Bid, construction documents provided and as stated in the Scope of Work.

All work must meet all applicable Federal, State, and local building codes and must be performed by qualified licensed contractors in their respective areas (electrical, mechanical, plumbing, fire, etc.). The Contractor must complete all work per Construction Documents including the associated specifications.

- LEAD TIME

Any long lead item must be ordered within one (1) week of issuance of project purchase order. Proof of purchase will be required.

- PERMIT REQUIREMENTS

Contractor is responsible for obtaining all necessary building permits and inspection approvals and paying all related costs/fees for the work included under this solicitation. Contractor is responsible to obtaining directly from the engineering firm at his own expense, if needed, any necessary engineering drawings (CAD drawings, signed/sealed electronic plans or hard copies). Contractor must provide copies of all permits obtained by Contractor to Parks and Water Resources, Facilities Management prior to the start of construction. Contractor responsible for installing a Permit Posting Box Unit. Approved plans and permits must be available on site for the inspector. For questions regarding building permits and review process please contact the Office Building Services at (352) 343-9653.

- PRODUCT HANDLING

Contractor is responsible for the proper packaging, shipping, handling and storage of materials to be incorporated in the work, so as to ensure the preservation of the quality and fitness of the materials for proper installation as required by the contract documents. Products are to be delivered to the site in the manufacturer’s container or packaging. Contractor shall provide the following: storage containers for material as necessary; sufficient manpower and equipment necessary to off load equipment/materials at jobsite per scheduled deliveries; and dumpsters for cardboard waste and packing debris. Contractor must store materials in a secure, weather-protected area and provide adequate security to protect delivered and stored products from theft, vandalism, or damage during the installation. Contractor is responsible for the return of all damaged products to manufacturer and the replacement of damaged items.

- PROPERTY REPLACEMENT

Property at the site, including turf (rubber, sod), concrete pavement, asphalt, lime rock path, and fencing removed or relocated during the course of construction and any damage due to the work performed by Contractor, must be reinstalled, repaired or replaced before final payment is made.

- SAFETY PRECAUTIONS

Contractor shall be responsible for initiating, maintaining, and supervising all safety-related precautions in connection with the work throughout the contract term. Contractor must erect and maintain all reasonable safeguards for safety and protection, including posting danger signs and other warning signs against hazards. It is the responsibility of the Contractor to provide a safe environment for Contractor’s employees, subcontractors, and all other users of the park, including patrons and County staff, and must monitor and maintain fencing/barricades to ensure continued safety.

- USE OF SITE

The Contractor must confine its operations at the site to areas permitted by law, ordinances, and permits. The Contractor must not unreasonably encumber the site with materials, equipment, or trailers. The Contractor shall not cause the site entrance to be blocked or otherwise prevent reasonable access to the site, other working and parking areas, completed portions of the work and/or properties, or storage areas. The Contractor will be given 24-hour notice by the County to remove or relocate any materials, equipment, or trailers improperly placed on the site. If Contractor fails to remove or relocate the items within the 24-hours provided, the County will remove the items at the Contractor's expense.

- UTILITIES

Contractor is responsible for making appropriate contact to ensure utility locations have been marked prior to excavation and trenching. Call 811 before you dig (www.Sunshine811.com). The Contractor must protect all utilities (including irrigation) encountered while performing its work. The Contractor must exercise due care when excavating around utilities and must restore any damaged utilities, at its own expense, to the same condition or better as existed prior to start of work.

- VENDOR SUBMITTAL

Submit product data/specifications and shop drawings electronically to Parks and Water Resources, Facilities Management for review and approval for all items of work. All submittals are to be approved by the engineer/architect and/or Parks and Water Resources, Facilities Management prior to Contractor delivering materials to job site and installation.

- AS-BUILT DRAWINGS

One (1) complete set of As-Built Drawings (signed/sealed by a licensed professional) in hard copy and electronic format along with operation and maintenance manuals (if applicable) shall be supplied to the County upon completion of the work and before submitting final payment application.

- WARRANTY AND GUARANTEES

Contractor must furnish Parks and Water Resources, Facilities Management with a signed minimum one (1) year warranty and maintenance program covering all systems from the date of final acceptance of the project by Lake County. The Contractor is responsible for securing warranties and guarantees for any materials, equipment, or fixtures to be incorporated into the project.

- WORKMANSHIP

  - All furnishings described in this solicitation must be installed by qualified tradesman. All installation work and materials to be per manufacturer’s specifications, or as directed by the County’s Representative.

  - All work and materials are subject to the approval of the County.

  - All products must be inspected by the County for damage and chipped or marred finish.  Contractor must replace any damaged or rejected products at no additional cost to the County. The County’s representative may at his option authorize acceptance of chipped or scratched painted surfaces repaired by Contractor at his own risk.

- EQUAL” PRODUCT CAN BE CONSIDERED UPON RECEIPT OF SPECIFIED DATA

  - Vendors must submit any equivalent request two (2) weeks prior to bid opening. Failure to submit timely will make equivalent submittal non-responsive.

  - The manufacturer's name, brand name and/or model number information contained in this solicitation are being used for the sole purpose of establishing the minimum requirement of level of quality, standard of performance, and design and is in no way intended to prohibit the offer of another manufacturer's items of equal material unless otherwise indicated elsewhere in this solicitation.

  - This specific solicitation requires submission of the following documentation to enable County evaluation of “equal” products:

______: Product Information Sheets

______: Product Samples with Initial Offer

______: Product Samples Upon Specific Request

______: Product labels

______: Performance Test Results

  - If an “equal” product may be considered by the County in accordance with this solicitation, the unit shall be equal in quality and standards of performance to the item specified in the solicitation. Where an “or equal” item is offered, and product information sheets are required, the initial offer must be accompanied with a complete set of product information sheets (such as factory specifications, standard manufacturer information sheets, catalogues, and brochures), and if required, a copy of performance test results of the unit offered as an equal. For product information submittals, all supporting documentation submitted by the vendor must in total meet the required specifications set forth in this solicitation. Where the standard product literature submitted with the offer provides information that does not comply with the specifications, the vendor shall state, in an official letter on corporate letterhead as part of their initial offer, the differences between the item they are specifically offering, and the equipment described by the standard product literature, to substantiate compliance to all of the specifications set forth in this solicitation. In such cases, any offer submitted with standard product literature but without the letter explaining compliance will result in the rejection of the offer for not meeting the solicitation specifications.

[End of Exhibit A.]